This file contains two summary tables for the parameterization of channels and channel properties at the HUC12 level for the SWAT+ model.  See accompanying publication for more details.

HUC12_Connect_revised.txt contains corrected HUC12 downstream connections.

Cha_Prop_2017 table

Contains representative channel data by HUC12 for the CONUS.

Field Description:

[HUC12] USGS Hydrologic Unit code 12 digit number .

[Stream_Order] Stream order as defined by NHDV2 Plus.

[Mean_Effective_Length_KM] Mean travel distance for runoff produced in the catchment.

[Mean_Slope] Mean slope in (m/m).

[Mean_Width] Mean width (m).

[Mean_Depth] Mean depth (m).

[Mean_Drainage_Area] Mean drainage area of channels in this group (km2).

[Drainage_Area_Incremental] Total incremental drainage area of channels in this group (km2).

[Drainage_Area_Total] Total drainage area for this group and all other contributing channels (km2).

[Channel_Count] Number of traced channels in this group.

[DA_Frac_Main] Fraction of the drainage area of this group which drains to the main channel.

[DA_Frac_2] Fraction of the drainage area of this group which drains to a second order channel (excludes main).

[DA_Frac_3] Fraction of the drainage area of this group which drains to a third order channel (excludes main).

[DA_Frac_4] Fraction of the drainage area of this group which drains to a forth order channel (excludes main).

[DA_Frac_5] Fraction of the drainage area of this group which drains to a fifth or highr order channel (excludes main).

[NHD_Segment_Count] Number of unique NHDV2Plus segments comprising this group.

[Note] Processing Notes

Trace_2017 table

Contains traced NHDV2Plus segments group into continuous channels of a given order.

Field Description:

[HUC12] USGS Hydrologic Unit code 12 digit number .

[COMID_Start] COMID of NHDV2Plus segment at start of the trace.

[COMID_End] COMID of NHDV2Plus segment at end of the trace.

[Stream_Order] Stream order as defined by NHDV2 Plus.

[Total_Length_KM] Total length of all segments in trace (km).

[Effective_Length_KM] Mean travel distance for runoff produced in the catchment (km).

[Slope] Mean slope in (m/m).

[Drainage_Area_Total] Total drainage area for this trace and all other contributing channels (km2)

[Drainage_Area_Incremental] Total incremental drainage area of channels in this trace (km2).

[Segment_Count] Number of unique NHDV2Plus segments comprising this trace.

[Drains_to_order] Stream order where this trace drains to.

[Drains_to_COMID] COMID where this trace drains to.